CSA1220 – COMPUTER ARCHITECTURE

CO2 Assessment Tool 1

# Question 1: 2's Complement Addition & Subtraction (−52 and +19)

Step 1: Convert both numbers to 8-bit binary

Positive numbers are written directly in binary; negative numbers are written in 2's complement (invert the bits of the magnitude and add 1).

+19  →  magnitude 19 = 00010011  →  since positive, +19 = 00010011

+52  →  magnitude 52 = 00110100

−52  →  invert 00110100 → 11001011  → add 1 → 11001100

So: −52 = 11001100 and +19 = 00010011 (both 8-bit signed values).

Step 2: Addition  (−52) + (+19)

1 1 0 0 1 1 0 0    (−52)

+  0 0 0 1 0 0 1 1    (+19)

---------------------

1 1 0 1 1 1 1 1

Adding bit by bit (with carries) gives the result 11011111.

Decoding 11011111 as a signed 2's complement number: invert → 00100000, add 1 → 00100001 = 33, so the value is −33.

Check: −52 + 19 = −33 ✔

Overflow check for addition

Overflow rule: overflow occurs only when two numbers of the SAME sign are added and the result's sign differs from theirs (equivalently, Carry-into-MSB XOR Carry-out-of-MSB = 1).

Here one operand is negative and one is positive, so overflow can never occur. Carry-in to bit7 = 0, Carry-out of bit7 = 0 → 0 XOR 0 = 0 → No overflow.

Step 3: Subtraction  (−52) − (+19)

Subtraction A − B is performed as A + (2's complement of B), i.e. (−52) + (−19).

+19 = 00010011 → −19: invert 11101100 → add 1 → 11101101

1 1 0 0 1 1 0 0    (−52)

+  1 1 1 0 1 1 0 1    (−19)

---------------------

1 0 1 1 1 0 0 1   (9-bit raw result, keep lowest 8 bits)

Keeping the 8-bit result: 10111001. Decoding: invert → 01000110, add 1 → 01000111 = 71, so the value is −71.

Check: −52 − 19 = −71 ✔

Overflow check for subtraction

Both operands being added internally (−52 and −19) are negative and the result (−71) is also negative — sign is consistent, so no overflow. Formally, Carry-into-MSB = 1 and Carry-out-of-MSB = 1 → 1 XOR 1 = 0 → No overflow. (A 9th carry bit is generated but discarded since we only keep 8 bits — this discarded carry is NOT overflow; overflow is a separate sign-based check.)

# Question 2: 8-bit Carry Look-Ahead Adder (CLA)

Step 1: Define Generate and Propagate signals

For each bit position i, define:

Generate:   Gi = Ai · Bi   (produces a carry regardless of incoming carry)

Propagate:  Pi = Ai ⊕ Bi   (passes an incoming carry through)

Example operands (8-bit): A = 10110101, B = 01101110

Bit-wise Gi and Pi values:

| Bit i | A | B | Gi=A·B | Pi=A⊕B |
| --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 | 1 |
| 2 | 1 | 1 | 1 | 0 |
| 3 | 0 | 1 | 0 | 1 |
| 4 | 1 | 0 | 0 | 1 |
| 5 | 1 | 1 | 1 | 0 |
| 6 | 0 | 1 | 0 | 1 |
| 7 | 1 | 0 | 0 | 1 |

Step 2: Compute all carries simultaneously using look-ahead equations

Instead of waiting for Ci-1 to ripple in, every carry is expressed directly in terms of G, P and C0 using two-level AND-OR logic:

C1 = G0 + P0C0

C2 = G1 + P1G0 + P1P0C0

C3 = G2 + P2G1 + P2P1G0 + P2P1P0C0

C4 = G3 + P3G2 + P3P2G1 + P3P2P1G0 + P3P2P1P0C0   (and so on up to C8)

Evaluating these with C0 = 0 for the example above gives:

| Carry | C0 | C1 | C2 | C3 | C4 | C5 | C6 | C7 | C8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |

Step 3: Compute sum bits and verify

Each sum bit is Si = Pi ⊕ Ci. Working this out for all 8 bits gives Sum = 00100011 with a final Carry-out C8 = 1.

Verification in decimal: A = 181, B = 110, A+B = 291. Since 291 > 255, the 8-bit result wraps to 291 − 256 = 35 = 00100011, with carry-out 1 — exactly matching the CLA computation ✔

Step 4: Compare execution time with a Ripple Carry Adder (RCA)

| Aspect | Ripple Carry Adder | Carry Look-Ahead Adder |
| --- | --- | --- |
| Carry generation | Sequential — each stage waits for Ci-1 | Parallel — all carries from G, P, C0 directly |
| Delay for n bits | O(n) gate delays (≈ n × 2-gate delay per FA) | O(log n) / near-constant per 4-bit block |
| Hardware cost | Low (simple, repeated full adders) | Higher (extra AND-OR carry logic) |
| Scalability | Delay grows linearly, poor for large n | Blocks can be cascaded hierarchically |

Step 5: Justification for using CLA in high-performance processors

- Carry propagation delay is the main speed bottleneck in an RCA; CLA removes this by computing every carry directly from the inputs.

- Because all carries are available almost simultaneously, the adder's delay becomes nearly independent of the operand width.

- This is essential in high-performance CPUs and ALUs where addition happens in almost every clock cycle (address calculation, ALU ops), so the extra gate cost of CLA logic is well justified by the large speed gain.

# Question 3: Booth's Algorithm — Multiply (+11) × (−7)

Step 1: Represent operands in 5-bit 2's complement

Multiplicand M = +11 = 01011        −M (2's complement of M) = 10101

Multiplier   Q = −7  = 11001

Initialize: A (Accumulator) = 00000, Q = 11001, Q-1 = 0. Five iterations are performed (one per bit).

Step 2: Apply Booth's rule each cycle

- Q0 Q-1 = 00 or 11  →  No arithmetic operation, only arithmetic right shift

- Q0 Q-1 = 10        →  A = A − M, then arithmetic right shift

- Q0 Q-1 = 01        →  A = A + M, then arithmetic right shift

Arithmetic right shift moves A, Q, Q-1 together right by one bit, keeping the sign bit of A unchanged (sign-extended).

Step 3: Iteration-by-iteration trace

| Iter | Q0 Q-1 | Operation | A (after op / after shift) | Q (after shift) | Q-1 |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 0 | A = A − M | 10101 → 11010 | 11100 | 1 |
| 2 | 0 1 | A = A + M | 00101 → 00010 | 11110 | 0 |
| 3 | 0 0 | No operation | 00010 → 00001 | 01111 | 0 |
| 4 | 1 0 | A = A − M | 10110 → 11011 | 00111 | 1 |
| 5 | 1 1 | No operation | 11011 → 11101 | 10011 | 1 |

Step 4: Final result

After 5 iterations: A = 11101, Q = 10011. The 10-bit product is A‖Q = 1110110011.

Decoding as a signed 2's complement number: invert → 0001001100, add 1 → 0001001101 = 77, so the value is −77.

Check: 11 × (−7) = −77 ✔

Step 5: How Booth's algorithm minimizes operations

- It scans pairs of bits (Q0, Q-1) instead of every single bit, so runs of consecutive 1s or 0s in the multiplier need only ONE addition/subtraction at their boundaries instead of one operation per bit.

- It handles negative multipliers directly in 2's-complement form without needing a separate sign-correction step, since the 10/01 transitions naturally encode the effective sign changes.

# Question 4: Division 25 ÷ 4 — Restoring vs Non-Restoring

Setup (common to both methods)

Using 5-bit registers: Dividend = 25 = 11001 (loaded into Q), Divisor M = 4 = 00100, Accumulator A = 00000 initially. n = 5 iterations are performed.

## Part A — Restoring Division

Rule applied every cycle

- Shift A,Q left by 1 bit.

- A = A − M.

- If A is negative → set new Q0 = 0 and RESTORE by A = A + M (undo the subtraction).

- If A is non-negative → set new Q0 = 1 and keep A as is.

| Iter | After shift (A,Q) | A − M | Sign | Action | Final A | Final Q |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 00001, 10010 | 11101 | −ve | Restore, Q0=0 | 00001 | 10010 |
| 2 | 00011, 00100 | 11111 | −ve | Restore, Q0=0 | 00011 | 00100 |
| 3 | 00110, 01000 | 00010 | +ve | Keep, Q0=1 | 00010 | 01001 |
| 4 | 00100, 10010 | 00000 | +ve | Keep, Q0=1 | 00000 | 10011 |
| 5 | 00001, 00110 | 11101 | −ve | Restore, Q0=0 | 00001 | 00110 |

Final: A (remainder) = 00001 = 1, Q (quotient) = 00110 = 6.

## Part B — Non-Restoring Division

Rule applied every cycle

- If A ≥ 0: shift A,Q left, then A = A − M.

- If A < 0: shift A,Q left, then A = A + M.

- After the add/subtract: if A ≥ 0 set Q0 = 1, else set Q0 = 0 (no restoring step needed here).

- After the last iteration, if A is negative, one final correction A = A + M is done to obtain the true remainder.

| Iter | A before | Shift → (A,Q) | Op | A after op | Sign | Q0 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 00000 (+ve) | 00001, 10010 | A−M | 11101 | −ve | 0 |
| 2 | 11101 (−ve) | 11011, 00100 | A+M | 11111 | −ve | 0 |
| 3 | 11111 (−ve) | 11110, 01000 | A+M | 00010 | +ve | 1 |
| 4 | 00010 (+ve) | 00100, 10010 | A−M | 00000 | +ve | 1 |
| 5 | 00000 (+ve) | 00001, 00110 | A−M | 11101 | −ve | 0 |

A ends negative (11101), so a final correction is applied: A = 11101 + 00100 (M) = 00001 = 1 (remainder). Quotient Q = 00110 = 6 (unchanged by the correction).

Both methods agree

Quotient = 00110 = 6, Remainder = 00001 = 1.  Check: 4 × 6 = 24, 25 − 24 = 1 ✔

Comparison

| Criterion | Restoring Division | Non-Restoring Division |
| --- | --- | --- |
| Operations/iteration | Subtract, and possibly an extra restoring add | Single add OR subtract only |
| Execution time | Slower — restore step adds extra cycles | Faster — no restore step per iteration |
| Hardware complexity | Simpler control logic | Slightly more complex control (tracks sign) |
| Efficiency | Lower, due to wasted restore operations | Higher — avoided operations reduce total additions |

# Question 5: IEEE 754 Representation and Floating-Point Addition

Step 1: Convert +26.75 to binary and normalize

26 = 11010,   0.75 = 0.11   →   26.75 = 11010.11

Normalized form: 1.101011 × 2^4   (sign = 0, exponent = 4)

Step 2: Single precision (32-bit) representation

Format: 1 sign bit | 8 exponent bits (bias 127) | 23 mantissa bits.

Sign = 0

Exponent = 4 + 127 = 131 = 10000011

Mantissa (fraction after leading 1) = 10101100000000000000000

32-bit pattern: 0 10000011 10101100000000000000000

Step 3: Double precision (64-bit) representation

Format: 1 sign bit | 11 exponent bits (bias 1023) | 52 mantissa bits.

Sign = 0

Exponent = 4 + 1023 = 1027 = 10000000011

Mantissa = 1010110000000000000000000000000000000000000000000000

Step 4: Floating-point addition of 26.75 + (−8.5)

Convert −8.5 to normalized binary: 8.5 = 1000.1 → 1.0001 × 2^3, so −8.5 = −(1.0001 × 2^3).

4a. Exponent alignment

The smaller exponent (3) must be raised to match the larger exponent (4). Shifting the mantissa of 8.5 right by 1 bit reduces its exponent by 1:

8.5 = 1.0001 × 2^3 = 0.10001 × 2^ (aligned)

4b. Mantissa operation

Since the signs differ (+26.75 and −8.5), the operation becomes a subtraction of magnitudes at the common exponent 2^4:

1.101011

− 0.100010

-----------

1.001001

4c. Normalization and rounding

The result 1.001001 × 2^4 already has a leading 1 (already normalized) and fits exactly within the available mantissa bits, so no rounding error is introduced.

4d. Convert back to decimal

1.001001 × 2^4 = 10010.01₂ = 18.25 (decimal)

Check: 26.75 + (−8.5) = 18.25 ✔

Effect of precision

In this case the result happens to be exactly representable, so single precision (23-bit mantissa) and double precision (52-bit mantissa) both give the exact answer 18.25 with no rounding error. In general, however, double precision retains far more mantissa bits, so it represents a much wider range of fractional values exactly and accumulates far less rounding error over repeated operations — which is why double precision is preferred for numerically sensitive engineering computations.